ECOTWIN

Reinforcement Learning for Urban Carbon Dispersal

Final Project Report

| Project Type | Advanced Data Science & ML Engineering |
| --- | --- |
| Contributors | Jerusha Jutike and Akash Vishwakarma |
| Core Stack | SUMO, TraCI, Python, Q-learning, FastAPI, WebSockets, React, Leaflet |
| Status | Final implementation completed and validated |

Prepared for final project review / demonstration

# Executive Summary

EcoTwin is an urban traffic and emissions simulation system that demonstrates how reinforcement learning can be used to control traffic signals while considering both mobility and environmental impact. The system uses SUMO to simulate traffic, TraCI to access vehicle and signal state, a tabular Q-learning controller to choose traffic-light actions, FastAPI for results and real-time WebSocket streaming, and a React/Leaflet dashboard for visual analytics.

The completed system compares a fixed-time traffic-light baseline against the trained RL controller over the same 200-second simulation horizon. After correcting the evaluation methodology so both controllers are measured on the same per-second basis, the RL controller reduced total waiting time by 40.73%, reduced accumulated CO2 emissions by 20.91%, and increased average vehicle speed by 19.19% in the project scenario.

# 1. Project Background and Problem Statement

Conventional traffic-signal optimization commonly focuses on traffic throughput and waiting time. EcoTwin extends this idea by also considering emissions. The project brief describes an urban planner viewing a simulated city grid where traffic-light decisions are driven by a reinforcement-learning agent and where carbon concentration is visualized as a live heatmap.

The central engineering problem is therefore multi-objective: improve traffic flow without ignoring the environmental cost of congestion. EcoTwin implements a reproducible simulation in which baseline and RL control strategies can be evaluated and visualized side by side.

# 2. Project Objectives

Create a reproducible urban traffic simulation using SUMO.

Implement a fixed-time traffic-light controller as the baseline.

Create an RL environment that exposes traffic state and applies signal actions.

Train and load a Q-learning controller for traffic-light control.

Measure waiting time, CO2 emissions, and average speed consistently.

Expose comparison results through a FastAPI REST endpoint.

Stream live simulation state through a WebSocket connection.

Render moving vehicles, live metrics, traffic-light state, and a carbon heatmap in React/Leaflet.

Validate the application end to end with repeatable backend and frontend checks.

# 3. Contributors and Scope

This final implementation was produced by two contributors: Jerusha Jutike and Akash Vishwakarma. Only these two contributors are included in this report and presentation material.

| Contributor | Primary responsibility areas |
| --- | --- |
| Jerusha Jutike | Project foundation; SUMO simulation setup; fixed-time baseline; RL environment and reward definition; Q-learning agent, training pipeline, and trained model; foundational frontend/Leaflet work; Docker and API configuration; integration groundwork. |
| Akash Vishwakarma | Simulation analytics UI; dashboard integration; corrected baseline-vs-RL evaluation methodology; live FastAPI WebSocket simulation stream; moving live vehicle visualization; carbon heatmap; backend hardening; duplicate evaluator cleanup; final documentation, verification, and integration. |

# 4. Requirement Mapping

| Project requirement | Final EcoTwin implementation |
| --- | --- |
| SUMO simulation environment | Implemented with a mock grid, routes, traffic-light junction, and TraCI-controlled execution. |
| RL traffic-light control | Implemented with a trained tabular Q-learning controller. |
| Multi-objective behavior | Reward/control work considers traffic flow and emissions; evaluation reports waiting time, CO2, and speed. |
| FastAPI + WebSockets | REST comparison endpoint plus /ws/simulation for live state streaming. |
| Live vehicle positions | Vehicle coordinates are streamed from SUMO and rendered as moving Leaflet markers. |
| Carbon heatmap | Live CO2 values feed a Leaflet heat layer. |
| Live signal state | Traffic-light phase is streamed and displayed. |
| Live/summary metrics | Dashboard displays live metrics and baseline-vs-RL charts/table. |

# 5. System Architecture

SUMO Traffic Simulation
        |
      TraCI
        |
  EcoTwin RL Environment
   /                \
Fixed-Time        Q-Learning
Baseline          Controller
   \                /
    Evaluation Results
          |
       FastAPI
     /         \
 REST JSON    WebSocket Stream
     \         /
       React Dashboard
          |
 Leaflet Map + Heatmap + Charts

# 6. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Simulation | SUMO 1.27.x | Traffic, vehicles, routes, signals, emissions. |
| Simulation interface | TraCI | Read live state and control traffic lights. |
| Machine learning | Python / Q-learning | Discrete traffic-light action policy. |
| Backend | FastAPI / Uvicorn | REST API and WebSocket server. |
| Frontend | React / Vite | Interactive dashboard. |
| Map | Leaflet / React Leaflet | Vehicle and city-grid visualization. |
| Heatmap | leaflet.heat | Live CO2 intensity visualization. |
| Charts | Recharts | Baseline-vs-RL result visualization. |
| Version control | Git / GitHub | Collaboration, PR review, and integration. |

# 7. Simulation and Baseline Controller

The SUMO environment models a small urban grid with traffic demand defined through route files. The fixed-time baseline uses the traffic-light timing already defined in the SUMO network. It does not make adaptive decisions; its purpose is to provide a reference against which the RL controller can be compared.

During evaluation, the baseline advances SUMO one second at a time and samples every active vehicle. CO2 emissions are accumulated every vehicle-second, speed samples are accumulated for the overall average, and each vehicle's latest accumulated waiting time is retained so the same waiting period is not repeatedly double-counted.

# 8. Reinforcement Learning Design

EcoTwin's final implementation uses tabular Q-learning. The project brief proposed PPO/DQN-style reinforcement learning, but the implemented prototype uses Q-learning because the traffic-light action space is small and discrete and because it provides a transparent controller that can be trained and demonstrated reliably in the project environment.

## 8.1 State and Action Flow

The environment obtains the current SUMO traffic state through TraCI.

The state is converted into the discrete RL state used by the Q-table.

The controller chooses the action with the highest learned Q-value for the current state.

The selected action changes the traffic-light phase.

SUMO advances and the process repeats.

## 8.2 Decision Interval

A new RL action is selected every 10 SUMO seconds. However, evaluation metrics are sampled every SUMO second. Separating the decision interval from the metric sampling interval is important because it keeps the baseline and RL measurements directly comparable.

## 8.3 Model Loading and Safety

The stored Q-learning model contains serialized state keys. The final loader parses those keys with ast.literal_eval() instead of Python eval(), preventing arbitrary expression execution while preserving the tuple-based state format.

# 9. Corrected Evaluation Methodology

A major final-stage correction was standardizing how the baseline and RL controller are evaluated. Earlier versions sampled the baseline every simulation second but sampled RL metrics only at its 10-second decision interval. This made the totals mathematically incomparable. The corrected implementation evaluates both systems for exactly 200 SUMO seconds and samples the same metrics every second.

| Metric | Definition used in final evaluation |
| --- | --- |
| Total waiting time | Sum of each vehicle's latest/final SUMO accumulated waiting time. |
| Total CO2 | Sum of instantaneous per-vehicle CO2 emission values sampled every SUMO second. |
| Average speed | Total speed across all vehicle-second observations divided by number of speed observations. |
| Horizon | 200 SUMO seconds for both baseline and RL. |

# 10. Backend and Real-Time Data Pipeline

FastAPI exposes the saved comparison results through a REST endpoint and provides a live WebSocket endpoint at /ws/simulation. When a WebSocket client connects, the backend starts the EcoTwin environment, loads the RL controller, advances the simulation, and sends a JSON snapshot after every simulation second.

simulation_time

vehicle_count

vehicle ID and SUMO x/y position

vehicle speed

vehicle CO2 emission rate

vehicle accumulated waiting time

current traffic-light phase

aggregate live CO2, waiting time, and average speed

The backend also validates the comparison-results JSON and returns meaningful HTTP errors if the file is missing, invalid, or incomplete.

# 11. Frontend Dashboard

The React dashboard combines post-simulation analytics with a live simulation map. Static placeholder vehicles were replaced by vehicle data arriving from the WebSocket stream.

## 11.1 Live Vehicle Map

SUMO produces local Cartesian coordinates rather than GPS coordinates. For the demonstration, EcoTwin normalizes the 0-300 simulation coordinate range into a small geographic display box around the Leaflet map center. This mapping is for visualization only and should not be interpreted as real-world GPS georeferencing.

## 11.2 Carbon Heatmap

Each live vehicle contributes a heat point whose intensity is derived from its current CO2 value. The heat layer uses green-to-yellow-to-orange-to-red visualization to indicate increasing emission intensity. The normalization threshold is a visualization heuristic for this simulated scenario, not a calibrated atmospheric concentration model.

## 11.3 Analytics

CO2 baseline-vs-RL chart

waiting-time baseline-vs-RL chart

average-speed baseline-vs-RL chart

comparison results table

RL impact summary

simulation details

live simulation state panel

# 12. Final Results

| Metric | Fixed-time baseline | RL controller | Change |
| --- | --- | --- | --- |
| Total waiting time | 248.00 s | 147.00 s | -40.73% |
| Total CO2 | 974,632.73 | 770,841.16 | -20.91% |
| Average speed | 4.9256 m/s | 5.8708 m/s | +19.19% |

For the evaluated scenario, the RL controller outperformed the fixed-time baseline on all three reported metrics. These results are simulation outputs for this configured network and demand pattern; they should not be generalized as real-world traffic-performance guarantees.

# 13. Validation and Testing

Baseline simulation completed for 200 seconds and reproduced the expected metrics.

RL evaluation completed for 200 seconds and reproduced the expected metrics.

Comparison generation produced the corrected percentage changes.

FastAPI backend started successfully.

REST comparison endpoint returned the corrected result JSON.

WebSocket stream delivered live vehicle positions and metrics.

Vehicle markers moved in the browser according to SUMO state.

Carbon heatmap rendered and updated during the simulation.

Frontend ESLint validation passed.

Vite production build passed.

Merged main branch passed the final end-to-end smoke test.

# 14. Individual Contributions

## 14.1 Jerusha Jutike

Created the EcoTwin project foundation and folder structure.

Set up and executed the initial SUMO traffic simulation.

Implemented the fixed-time traffic baseline and baseline result model.

Built the EcoTwin RL environment, reset behavior, and normalized reward logic.

Defined the RL problem and connected the simulation environment to the controller.

Implemented the Q-learning agent and training pipeline.

Generated and saved the trained Q-learning model.

Developed foundational frontend work, including React/Leaflet setup and dashboard styling.

Added Docker deployment and frontend API configuration.

Performed core integration work that connected the project modules into a working application.

## 14.2 Akash Vishwakarma

Built and polished the simulation analytics components for emissions, waiting time, speed, comparison results, RL impact, and simulation details.

Integrated the analytics into the final UrbanMap dashboard.

Audited the evaluation implementation and identified mismatched sampling intervals and waiting-time double-counting.

Standardized baseline and RL evaluation to the same 200-second, per-second metric methodology.

Implemented the FastAPI WebSocket live SUMO stream.

Replaced hard-coded map markers with real live SUMO vehicle positions and data.

Implemented the live carbon heatmap using leaflet.heat.

Hardened backend result validation and Q-learning model parsing.

Removed the stale duplicate RL evaluator.

Prepared final documentation, repeated validation, pull-request integration, and final main-branch verification.

Both contributors understand the complete pipeline and can explain the other contributor's work at system level. The division above identifies primary ownership rather than implying that either contributor worked in isolation.

# 15. Known Limitations

The implemented RL algorithm is tabular Q-learning rather than PPO/RLlib.

The SUMO network is a small synthetic grid rather than a calibrated real city network.

The Leaflet coordinates are a visual mapping of local SUMO coordinates, not true GPS coordinates.

The carbon heatmap represents simulated per-vehicle CO2 intensity, not a physical atmospheric dispersion model.

The current WebSocket design assumes one active TraCI simulation per backend process.

The fixed 200-second evaluation horizon can leave a vehicle active in one controller while none remain in the other; the comparison is therefore a fixed-horizon controller comparison.

# 16. Future Enhancements

Replace tabular Q-learning with PPO or DQN and compare policies under the same evaluation protocol.

Use a larger, georeferenced SUMO network and realistic traffic demand.

Model spatial atmospheric dispersion rather than only per-vehicle emission intensity.

Add throughput/completed-trip metrics and explicit remaining-vehicle reporting.

Run multiple random seeds and report mean, standard deviation, and confidence intervals.

Separate simulation workers from WebSocket broadcasting for multi-user deployment.

Add persistent experiment tracking and model-version metadata.

# 17. Conclusion

EcoTwin demonstrates a complete simulation-to-dashboard ML engineering workflow. Traffic is generated in SUMO, controlled through TraCI, optimized by a trained RL policy, evaluated against a fixed-time baseline, served through FastAPI, streamed live over WebSockets, and visualized in React/Leaflet. The final evaluation shows meaningful gains in waiting time, CO2, and average speed for the configured simulation. Equally important, the final implementation corrects the evaluation methodology so those gains are measured consistently.

# References

EcoTwin project brief: Project 3 - “EcoTwin”: Reinforcement Learning for Urban Carbon Dispersal.

Eclipse SUMO documentation - Simulation of Urban MObility and TraCI.

FastAPI documentation - REST APIs and WebSockets.

React and React Leaflet documentation.

Sutton, R. S., & Barto, A. G. Reinforcement Learning: An Introduction, 2nd ed.